Presto-Lock Coping

SECTION 07710

I.	PART ONE GENERAL

1.01	SUMMARY:

- Work included: Furnishing and installing factory fabricated and finished coping system.

- Related work: [Insert applicable sections].

1.02	REFERENCES:

- FM Approvals, 1151 Boston-Providence Turnpike, PO Box 9102, Norwood, MA 02082 USA, Phone: 781-762-4300 (Main), Fax: 781-762-9375 (Main), Web: www.fmglobal.com.

- SPRI Single Ply Roofing Industry, 77 Rumford Ave., Suite 3B, Waltham, MA 02453, Phone: 781-647-7026, Fax: 781-647-7222, E-mail: info@spri.org, Web: www.spri.org.

1.03	SUBMITTALS:

- Product Data: Provide manufacturer’s product and complete installation data for all materials in this specification.

- Shop Drawings: Show profiles, joining method, accessory location, anchorage and flashing details, adjacent construction interface, and dimensions.

- Samples: Available upon request; sized to represent material adequately.

- Installation Guide: The product manufacturer shall provide a written installation guide.

1.04	QUALITY ASSURANCE:

- High performance coping  shall be CERTIFIED by the manufacturer to meet performance design criteria according to the following test standards: [select, if applicable]:

  - ANSI/SPRI ES-1 Test Method RE-3 for Coping: Wind Design Standard for Edge Systems Used with Low Slope Roofing Systems (current edition). The coping system shall be tested simultaneously on horizontal and vertical surfaces and shall exceed horizontal and vertical design wind pressure as calculated in accord with the ANSI/SPRI ES-1 Test RE-3. Use the current edition of ANSI/SPRI ES-1 Wind Design Standard for Edge Systems Used with Low Slope Roofing Systems.

  - The coping product shall be listed in current Factory Mutual Research Corporation Approval Guide approved for Class FM 1-90.

1.05	PRODUCT HANDLING:

- All material shall arrive in the manufacturer’s original sealed, labeled containers.

- Store materials in a dry, protected, well-vented area. The contractor shall report damaged material immediately to the delivering carrier and note such damage on the carrier’s freight bill of lading.

- Remove protective plastic surface film immediately after installation [if applicable].

1.06	SUBSTITUTIONS:

- Proposals for substitution products shall be accepted only from bidding contractors a minimum of 10 working days before bid due date. The proposed substitution shall meet the performance and quality standards of this specification (see section 1.04).

1.07	JOB CONDITIONS:

- Verify that other trades with related work are complete before mounting coping covers.

- Mounting surfaces shall be straight and secure; substrates shall be of proper width.

- Refer to the construction documents, shop drawings and manufacturer’s installation instructions.

- Coordinate installation with roof membrane manufacturer’s installation instructions before starting.

- Observe all appropriate OSHA safety guidelines for this work.

1.08	WARRANTY/GUARANTEE:

- Provide a Johns Manville full system warranty for roof edge systems, when installed per Johns Manville’s instructions.

- Provide a 20 year warranty for painted finish covering color fade, chalk and film integrity.

II.	PART TWO PRODUCTS

2.01	MANUFACTURER:

A.	Johns Manville

1600 Airport Road

Waukesha, WI 53188

262-549-6900

Fax: 262-549-6009

Internet address: www.metalera.com/manville

2.02	PARAPET COPING SYSTEM:

- Presto-Lock Coping: Metal coping cap with galvanized steel anchor/support cleats for capping any parapet wall. The system shall be watertight, maintenance free, and does not require exposed fasteners. Joints shall be a butt type with concealed splice plates. Standard model is (PTCJ) for all sizes.

- Performance Characteristics:

- Coping sections shall expand and contract freely while locked in place on anchor cleats.

- Coping sections shall lock to anchor cleats by mechanical pressure from hardened stainless steel springs factory attached to anchor cleats.

- All splice plates include factory applied dual non-curing sealant strips capable of providing a watertight seal.

- Metal [choose]: (.040", .050" or .063" aluminum; 24 ga. or 22 ga. galvanized steel with Kynar coatings; custom metal by request).

- Coping cap: length of 12'-0" (3.65 m), widths to 24" manufactured to job requirements. True radii may be built to template.

- Coping vertical face and back leg: 2¼" to 12½" manufactured to job requirements.

- Concealed splice plates: 8" wide. Finish to match finish of coping cap with factory applied dual non-curing sealant strips.

- Anchor/Support Cleat: 20 ga. prepunched galvanized cleat with stainless steel spring mechanically locked to cleat normally 12” (304.8 mm) wide @ 4’-0” (1.22 mm) on center. Mechanically fastened as indicated and detailed.

- Fasteners: #9 x1½" stainless steel fasteners provided with drivers. No exposed fasteners shall be permitted. Fasteners shall be electrolytically compatible.

- Finishes: Shall be [select: natural aluminum mill finish; standard pre-coated Kynar 500 from manufacturer’s color list; standard post-coated Kynar 500 from industry standard two-coat colors; non-standard Kynar 500 color with multiple coatings or protective clear coats; anodized aluminum; post applied exterior finish process] color shall be [insert color].

2.03	ACCESSORIES:

- Corners, end caps, pier caps, etc. shall be fabricated by the coping manufacturer.

- Welded or quicklock assembly shall be used to maintain watertight integrity.

II.	PART THREE EXECUTION

3.01	INSPECTION:

- Verify that coping installation will not disrupt other trades. Verify that substrate is dry, clean and free of foreign matter. Report and correct defects prior to any installation.

3.02	INSTALLATION:

- Submit product design drawings for review and approval to Architect or Specifier before fabrication.

- Installing contractor shall check as-built conditions and verify Johns Manville’s coping details for accuracy to fit the wall assembly prior to fabrication. The installer shall comply with Johns Manville’s installation guide when setting copings.

- Installer shall use provided fasteners consistent with Johns Manville’s instructions suitable for the substrate to which it is being installed.

END OF SECTION 07710