Seal-Tite Industrial Gutter

SECTION 07710

I. PART ONE GENERAL

1.01 SUMMARY:

- Work included: Furnishing and installing factory fabricated and finished roof edging.

- Related Work: [Insert applicable sections].

1.02 REFERENCES:

- Factory Mutual Research Corporation (FMRC), P.O. Box 9102, Norwood, MA 02082, 617-762-4300.

- SPRI Sheet Membrane & Component Suppliers to the Commercial Roofing Industry, 175 Highland Ave., Needham, MA 02194, 617-444-0242, fax: 617-444-6111.

1.03 SUBMITTALS:

- Product Data: Provide manufacturer’s product and complete installation data for all materials in this specification.

- Shop Drawings: Show profiles, joining method, location of accessory items, anchorage and flashing details, adjacent construction interface, and dimensions.

- Samples: Available upon request; size to adequately represent material.

- Installation Guide: The product manufacturer shall provide a written installation guide.

1.04 QUALITY ASSURANCE:

- The product shall be listed in current Factory Mutual Research Corporation Approval Guide approved for [FM 1-90].

1.05 PRODUCT HANDLING:

- All material shall arrive in the manufacturer’s original sealed, labeled containers.

- Store materials in a dry, protected, well-vented area. The contractor shall report damaged material immediately to the delivering carrier and note such damage on the carrier’s freight bill of lading.

- Remove protective plastic surface film immediately before installation.

1.06 SUBSTITUTIONS:

- Proposals for substitution products shall be accepted only from bidding contractors a minimum of 10 working days before bid due date. The proposed substitution shall meet the performance and quality standards of this specification (see section 1.04).

1.07 JOB CONDITIONS:

- Verify that other trades are complete before installing the roof edging.

- Mounting surfaces shall be straight and secure; substrates shall be of proper width.

- Refer to the construction documents, shop drawings and manufacturer’s installation instructions.

- Coordinate installation with roof membrane manufacturer’s installation instructions.

- Observe all appropriate OSHA safety guidelines for this work.

1.08 WARRANTY/GUARANTEE:

- Manufacturer’s Standard Warranty: Warranted materials shall be free of defects in material and workmanship for five years after shipment. If, after inspection, the manufacturer agrees that materials are defective, the manufacturer shall, at their option, repair or replace them. For decorative finish warranty, consult manufacturer.

- Special Performance/20-Year Warranty [optional]: Manufacturer shall guarantee that a standard size roof edge system, when installed per manufacturer’s instructions, will not blow off or cause membrane failure, even in wind conditions up to 110 mph, or the manufacturer shall replace or repair their materials.

II. PART TWO PRODUCTS

2.01 MANUFACTURER:

A. 	Metal-Era, Inc.

1600 Airport Road

Waukesha, WI 53188

262-549-6900

Fax: 262-549-6009

Internet address: www.metalera.com

2.02 ROOF EDGING SYSTEM:

- Seal-Tite Gutter System: Designed to accommodate the drainage of large roof areas. Model(s) shall be [choose one].

  - Three standard designs: (IG-1, IG-2, and IG-B) available to accommodate most architectural requirements. Custom sizes by request. Radius gutters available on in IG-B.

- PERFORMANCE CHARACTERISTICS:

  - Heavy gauge gutter straps securely support large volumes of water, as well as extreme snow and icing conditions.

  - Manufactured to rigid tolerances and furnished per required drainage capacity/size.

  - Adapts easily to “optional” drainage bars or flow through gravel stops.

- Gutter metal gauge [choose]: .040", .050", or .063" thick formed aluminum; or 22 gauge or 24 gauge galvanized steel with Kynar 500 finish.

- Gutter: standard 12'-0" (3.65 m) lengths.

- Exterior gutter finishes: [For aluminum choose] Natural mill finish, Kynar 500 from manufacturer’s standard colors, custom color Kynar 500, or Clear or Color Anodized. [For 22 gauge or 24 gauge galvanized steel choose] Kynar 500 standard color from manufacturer’s standard colors.

2.03 ACCESSORIES:

- Corners, end caps, expansion joints or exterior brackets shall be fabricated by manufacturer. Factory fabricated, mitered corners shall have 17½" nominal leg lengths.

- Provide matching ledge caps, downspouts, or other special fabrications as detailed.

III. PART THREE EXECUTION

3.01 INSPECTION:

- Verify that the roof edging installation will not disrupt other trades. Verify that the substrate is dry, clean and free of foreign matter. Report and correct defects prior to any installation.

3.02 INSTALLATION OF SEAL-TITE GUTTER SYSTEM:

- Submit product design drawings for review and approval to Architect or Specifier before fabrication.

- Installing contractor shall check as-built conditions and verify the manufacturer’s gravel stop details for accuracy to fit the wall assembly prior to fabrication. The installer shall comply with the roof edging manufacturer’s installation guide when setting edging.

- Installer shall furnish mechanical fasteners consistent with manufacturer’s instructions; suitable for the substrate to which being installed.

END OF SECTION 07710