Presto-Tite Canted Fascia

SECTION 07710

I.	PART ONE GENERAL

1.01	SUMMARY:

- Work included: Furnishing and installing factory fabricated and finished roof edging.

- Related Work: [Insert applicable sections].

1.02	REFERENCES:

- FM Approvals, 1151 Boston-Providence Turnpike, PO Box 9102, Norwood, MA 02082 USA, Phone: 781-762-4300 (Main), Fax: 781-762-9375 (Main), Web: www.fmglobal.com.

- SPRI Single Ply Roofing Industry, 77 Rumford Ave., Suite 3B, Waltham, MA 02453, Phone: 781-647-7026, Fax: 781-647-7222, E-mail: info@spri.org, Web: www.spri.org.

1.03	SUBMITTALS:

- Product Data: Provide manufacturer’s product and complete installation data for all materials in this specification.

- Shop Drawings: Show profiles, joining method, location of accessory items, anchorage and flashing details, adjacent construction interface, and dimensions.

- Samples: Available upon request; size to adequately represent material.

- Installation Guide: The product manufacturer shall provide a written installation guide.

1.04	QUALITY ASSURANCE:

- High performance roof edge system shall be CERTIFIED by the manufacturer to comply with ANSI/SPRI Standard ES-1. Roof edge shall meet performance design criteria according to the following test standards: [select, if applicable]:

  - ANSI/SPRI ES-1 Test Method RE-1 Test for Roof Edge Termination of Single-Ply Roofing Membranes: The fascia system shall be tested to secure the membrane to minimum of 100 lbs/ft in accord with the ANSI/SPRI ES-1 Test Method RE-1. Use the current edition of ANSI/SPRI ES-1 Wind Design Standard for Edge Systems Used with Low Slope Roofing Systems.

  - ANSI/SPRI ES-1 Test Method RE-2 Pull-Off Test for Fascia: The fascia system shall be tested in accord with the ANSI/SPRI ES-1 Test Method RE-2. Use the current edition of ANSI/SPRI ES-1 Wind Design Standard for Edge Systems Used with Low Slope Roofing Systems.

- The fascia product shall be listed in current Factory Mutual Research Corporation Approval Guide. Approved for [FM 1-270].

- The fascia product shall be approved for use in Miami-Dade County and has been designed to comply with Florida Building Code, including the High Velocity Hurricane Zone. Miami-Dade County NOA

No. 03-0108.06 Expiration Date: 12/11/08.

1.05	PRODUCT HANDLING:

- All material shall arrive in the manufacturer’s original sealed, labeled containers.

- Store materials in a dry, protected, well-vented area. The contractor shall report damaged material immediately to the delivering carrier and note such damage on the carrier’s freight bill of lading.

- Remove protective plastic surface film immediately before installation.

1.06	SUBSTITUTIONS:

- Proposals for substitution products shall be accepted only from bidding contractors a minimum of 10 working days before bid due date. The proposed substitution shall meet the performance and quality standards of this specification (see section 1.04).

- Where pre-engineered manufactured systems are specified, other field fabricated or shop/field-fabricated substitutions will not be accepted. However, where shop/field fabrications are indicated, pre-engineered systems will be considered with architect approval.

1.07	JOB CONDITIONS:

- Verify that other trades are complete before installing the roof edging.

- Mounting surfaces shall be straight and secure; substrates shall be of proper width.

- Refer to the construction documents, shop drawings and manufacturer’s installation instructions.

- Coordinate installation with roof membrane manufacturer’s installation instructions.

- Observe all appropriate OSHA safety guidelines for this work.

1.08	WARRANTY/GUARANTEE:

- Provide a Johns Manville full system warranty for roof edge system when installed per Johns Manville’s instructions.

- Provide a 20 year warranty for painted finish covering color fade, chalk, and film integrity.

II.	PART TWO PRODUCTS

2.01	MANUFACTURER:

A.	Johns-Manville

1600 Airport Road

Waukesha, WI 53188

262-549-6900

Fax: 262-549-6009

Internet address: www.metalera.com/manville

2.02	ROOF EDGING SYSTEM:

- Presto-Tite: Shall be a three-part assembly designed for adhered or mechanically attached single ply roofs, for Modified or Built-Up roof applications and for existing beveled wood nailer applications: Consisting of a continuous extruded aluminum bar, exterior fascia and continuous formed galvanized steel cleat with fasteners. To terminate adhered or mechanically attached single-ply roofing at perimeter. The system shall be watertight with no exposed fasteners. Model(s) shall be (choosing from the following sizes): PEFC-552, PEFC-902, PEFC -1052, PEFC -1202, PEFC -1352 for single ply, modified or built-up roof applications, and  PEFC-55, PEFC-900, PEFC-1050, PEFC-1200, PEFC-1350 for existing beveled wood nailer applications. The rise above the nailer for single-ply, modified and built-up roof applications the rise is 2". Fascia model number denotes inches; (i.e. PFEC-1052 has a 10½" face height). Non-standard sizes are available upon request.

- PERFORMANCE CHARACTERISTICS:

  - Extruded bar shall lock membrane, prevent wind pullback.

  - Fascia shall freely thermal cycle on extruded bar, preventing periodic maintenance.

  - Fascia may be factory modified for true radius application.

- Fascia metal gauge (choose): .050" or .063" thick-formed aluminum with Kynar 500 finish.

- Fascia: standard 12'-0" (3.65 m) lengths with matching concealed joint splice plates.

- Extruded Bar: Shall be continuous 6063-T6 alloy aluminum in 12'-0" (3.65 m) standard lengths with pre-punched slotted holes. All bar miters are welded.

- Waterdam: Standard 12'-0" (3.65 M) lengths of commercial type 24 ga. G-90 galvanized steel with pre-punched slotted holes.

- Anchor Bar Cleat: Standard 12'-0" (3.65 M) lengths of commercial type 20 ga. G-90 galvanized steel with pre-punched holes.

- Leveler: Standard 4" wide leveler of commercial type 24 ga. G-90 galvanized steel.

- Fasteners: #9 x 2" stainless steel fasteners provided with drivers. No exposed fasteners permitted.

- Fasteners: ¼" x 1¼" masonry/wood fasteners provided with drivers. No exposed fasteners permitted.

- Fasteners:  1¼" ring shank nails provided. No exposed fasteners permitted.

- Exterior fascia finishes: (For aluminum choose) natural mill finish, Kynar 500 from manufacturer’s standard colors, custom color Kynar 500, or clear or color anodized.

2.03	ACCESSORIES:

- Miters, downspout scuppers, spillout scuppers shall be fabricated by manufacturer.

- Welded base assembly shall be used to maintain watertight integrity.

- Provide matching brick wall cap, downspout, extenders, or other special fabrications as detailed.

III.	PART THREE EXECUTION

3.01	INSPECTION:

- Verify that the roof edging installation will not disrupt other trades. Verify that the substrate is dry, clean and free of foreign matter. Report and correct defects prior to any installation.

3.02	INSTALLATION OF PRESTO-TITE ROOF EDGING SYSTEM:

- Submit product design drawings for review and approval to Architect or Specifier before fabrication.

- Installing contractor shall check as-built conditions and verify the manufacturer’s roof edge details for accuracy to fit the wall assembly prior to fabrication. The installer shall comply with the roof edging manufacturer’s installation guide when setting edging.

- Installer shall use provided fasteners consistent with manufacturer’s instructions, suitable for the substrate to which it is being installed.

END OF SECTION 07710