SECTION 07710

# STEVENS FASCIA FOR 4” TO 8-1/2” FACE HEIGHTS IN 24 GA. & .040” MATERIAL

## PART ONE GENERAL

  - SUMMARY:

- Work included: Furnishing and installing factory fabricated and finished roof edging.

- Related Work: [Insert applicable sections].

  - REFERENCES:

- Factory Mutual Research Corporation (FMRC), P.O. Box 9102, Norwood, MA 02082, 617-762-4300.

- SPRI Sheet Membrane & Component Suppliers to the Commercial Roofing Industry, 175 Highland Ave., Needham, MA 02194, 617-444-0242, fax: 617-444-6111.

  - SUBMITTALS:

- Product Data: Provide manufacturer’s product and complete installation data for all materials in this specification.

- Shop Drawings: Show profiles, joining method, location of accessory items, anchorage and flashing details, adjacent construction interface, and dimensions.

- Samples: Available on request; size to adequately represent material.

- Installation Guide: The product manufacturer shall provide a written installation guide.

  - QUALITY ASSURANCE:

- High performance roof edge system shall be CERTIFIED by the manufacturer to comply with ANSI/SPRI Standard ES-1-98. Roof edge shall meet performance design criteria according to the following test standards: [select, if applicable]:

1.	ANSI/SPRI ES-1-98 Test Method RE-1 Test for Roof Edge Termination of Single-Ply Roofing Membranes: The fascia system shall be tested to secure the membrane to minimum of 100 lbs/ft in accord with the ANSI/SPRI ES-1-98 Test Method RE-1. Use the current edition of ANSI/SPRI ES-1 Wind Design Standard for Edge Systems Used with Low Slope Roofing Systems.

- ANSI/SPRI ES-1-98 Test Method RE-2 Pull-Off Test for Fascia: The fascia system shall be tested in accord with the ANSI/SPRI ES-1-98 Test Method RE-2. Use the current edition of ANSI/SPRI ES-1 Wind Design Standard for Edge Systems Used with Low Slope Roofing Systems.

- The fascia product shall be listed in current Factory Mutual Research Corporation Approval Guide approved for [FM 1-645].

- The fascia product shall be approved for use in Miami-Dade County and has been designed to comply with Florida Building Code, including the High Velocity Hurricane Zone. Miami-Dade County NOA No. 03-0108.06 Epiration Date: 12/11/08.

  - PRODUCT HANDLING:

- All material shall arrive in the manufacturer’s original sealed, labeled containers.

- Store materials in a dry, protected, well-vented area. The contractor shall report damaged material immediately to the delivering carrier and note such damage on the carrier’s freight bill of lading.

- Remove protective plastic surface film immediately before installation.

  - SUBSTITUTIONS:

- Proposals for substitution products shall be accepted only from bidding contractors a minimum of 10 working days before bid due date. The proposed substitution shall meet the performance and quality standards of this specification (see section 1.04).

- Where pre-engineered manufactured systems are specified, other field fabricated or shop/field-fabricated substitutions will not be accepted. However, where shop/field fabrications are indicated, pre-engineered systems will be considered with architect approval.

  - JOB CONDITIONS:

- Verify that other trades are complete before installing the roof edging.

- Mounting surfaces shall be straight and secure; substrates shall be of proper width.

- Refer to the construction documents, shop drawings and manufacturer’s installation instructions.

- Coordinate installation with roof membrane manufacturer’s installation instructions.

- Observe all appropriate OSHA safety guidelines for this work.

  - WARRANTY/GUARANTEE:

- Provide a Stevens full system warranty for roof edge system, when installed per Steven’s instructions, covering blow-off from winds up to 150 mph.

- Provide a 20-year warranty for painted finish covering color fade, chalk, and film integrity.

## PART TWO PRODUCTS

  - MANUFACTURER:

- Stevens Roofing Systems

1600 Airport Road

Waukesha, WI 53188

1-262-549-6900

Fax: 1-262-549-6009

Internet address: www.metalera.com/stevens

  - ROOF EDGING SYSTEM:

- Stevens Fascia: Decorative metal fascia with continuous extruded aluminum bar. To terminate adhered or mechanically attached single-ply roofing at perimeter. The system shall be watertight with no exposed fasteners. Model(s) shall be (choose from the following sizes): SB-40, SB-55, SB-70, SB-85. The rise above the nailer for all models is 1-1/4”. Fascia model number denote inches; (i.e. SB-55 has a 5-1/2” face height).

- PERFORMANCE CHARACTERISTICS:

- Extruded bar shall lock membrane, prevent wind pullback.

- Injection molded EPDM splices to allow thermal expansion of extruded aluminum anchor bar.

- Fascia shall freely thermal cycle on extruded bar, preventing periodic maintenance.

- Fascia may be factory modified for true radius application.

- Fascia metal gauge (choose): .040” thick formed aluminum or 24 ga. galvanized steel with Kynar 500 finish.

- Fascia: standard 12’-0” (3.65 m) lengths.

- Extruded bar: Shall be continuous 6063-T6 alloy aluminum at 12’-0” (3.65 m) standard lengths. All bar miters are welded.

- Fasteners: #12 x 1-5/8” corrosion resistant fasteners provided with drivers. No exposed fasteners permitted.

- Exterior fascia finishes: (For aluminum choose) natural mill finish, Kynar 500 from manufacturer’s standard colors, custom color Kynar 500, or clear or color anodized. (For 24 gauge galvanized steel choose) Kynar 500 standard color from manufacturer’s standard colors.

  - ACCESSORIES:

- Miters, downspout scuppers, and spillout scuppers shall be fabricated by manufacturer.

- Welded base assembly shall be used to maintain watertight integrity.

- Provide matching brick wall cap, downspout, extenders, or other special fabrications as detailed.

## PART THREE EXECUTION

  - INSPECTION:

- Verify that the roof edging installation will not disrupt other trades. Verify that the substrate is dry, clean and free of foreign matter. Report and correct defects prior to any installation.

  - INSTALLATION OF STEVENS FASCIA SYSTEM:

- Submit product design drawings for review and approval to Architect or Specifier before fabrication.

- Installing contractor shall check as-built conditions and verify the Steven’s roof edge details for accuracy to fit the wall assembly prior to fabrication. The installer shall comply with Steven’s installation guide when setting edging.

- Installer shall use provided fasteners consistent with Steven’s instructions, suitable for the substrate to which it is being installed.

- Install Steven’s all-purpose sealant, as recommended by the membrane manufacturer, under the anchor bar.

END OF SECTION 07710