Presto-Tite Fascia: Fully Adhered or Mechanically Attached Single-Ply Version
AND
Presto-Tite Fascia: Stone Ballast Single-Ply Version

SECTION 07710

I.	PART ONE GENERAL

1.01	SUMMARY:

- Work included: Furnishing and installing factory fabricated and finished roof edging.

- Related Work: [Insert applicable sections].

1.02	REFERENCES:

- FM Approvals, 1151 Boston-Providence Turnpike, PO Box 9102, Norwood, MA 02082 USA, Phone: 781-762-4300 (Main), Fax: 781-762-9375 (Main), Web: www.fmglobal.com.

- SPRI Single Ply Roofing Industry, 77 Rumford Ave., Suite 3B, Waltham, MA 02453, Phone: 781-647-7026, Fax: 781-647-7222, E-mail: info@spri.org, Web: www.spri.org.

1.03	SUBMITTALS:

- Product Data: Provide manufacturer’s product and complete installation data for all materials in this specification.

- Shop Drawings: Show profiles, joining method, location of accessory items, anchorage and flashing details, adjacent construction interface, and dimensions.

- Samples: Available on request; size to adequately represent material.

- Installation Guide: The product manufacturer shall provide a written installation guide.

1.04 QUALITY ASSURANCE:

- High performance roof edge system shall be CERTIFIED by the manufacturer to comply with ANSI/SPRI Standard ES-1. Roof edge shall meet performance design criteria according to the following test standards: [select, if applicable]:

  - ANSI/SPRI ES-1 Test Method RE-1 Test for Roof Edge Termination of Single Ply Roofing Membranes: The fascia system shall be tested to secure the membrane to minimum of 100 lbs/ft in accord with the ANSI/SPRI ES-1 Test Method RE-1. Use the current edition of ANSI/SPRI ES-1 Wind Design Standard for Edge Systems Used with Low Slope Roofing Systems.

  - ANSI/SPRI ES-1 Test Method RE-2 Pull-Off Test for Fascia: The fascia system shall be tested in accord with the ANSI/SPRI ES-1 Test Method RE-2. Use the current edition of ANSI/SPRI ES-1 Wind Design Standard for Edge Systems Used with Low Slope Roofing Systems.

- The fascia product shall be listed in current Factory Mutual Research Corporation Approval Guide approved for [FM 1-645].

1.05	PRODUCT HANDLING:

- All material shall arrive in the manufacturer’s original sealed, labeled containers.

- Store materials in a dry, protected, well-vented area. The contractor shall report damaged material immediately to the delivering carrier and note such damage on the carrier’s freight bill of lading.

- Remove protective plastic surface film immediately before installation.

1.06	SUBSTITUTIONS:

- Proposals for substitution products shall be accepted only from bidding contractors a minimum of 10 working days before bid due date. The proposed substitution shall meet the performance and quality standards of this specification (see section 1.04).

- Where pre-engineered manufactured systems are specified, other field fabricated or shop/field-fabricated substitutions will not be accepted. However, where shop/field fabrications are indicated, pre-engineered systems will be considered with architect approval.

1.07	JOB CONDITIONS:

- Verify that other trades are complete before installing the roof edging.

- Mounting surfaces shall be straight and secure; substrates shall be of proper width.

- Refer to the construction documents, shop drawings and manufacturer’s installation instructions.

- Coordinate installation with roof membrane manufacturer’s installation instructions.

- Observe all appropriate OSHA safety guidelines for this work.

1.08	WARRANTY/GUARANTEE:

- Provide a Johns Manville full system warranty for roof edge system when installed per Johns Manville’s instructions.

- Provide a 20 year warranty for painted finish covering color fade, chalk, and film integrity.

II.	PART TWO PRODUCTS

2.01	MANUFACTURER:

A.	Johns Manville

1600 Airport Road

Waukesha, WI 53188

262-549-6900

Fax: 262-549-6009

Internet address: www.metalera.com/manville

2.02	ROOF EDGING SYSTEM:

- Presto-Tite Fascia: Decorative metal fascia with continuous extruded aluminum bar. To terminate adhered, ballasted or mechanically attached single-ply roofing at perimeter. The system shall be watertight with no exposed fasteners. Model(s) shall be for adhered or mechanically attached (choose from the following sizes): PF-40, PF-55, PF-70, PF-85. The rise above the nailer for all models is 1¼". Fascia model number denote inches; (i.e. PF-55 has a 5½" face height). Model(s) shall be for the ballasted, (choose from the following sizes): PFB-50, PFB-65, PFB-80. This rise above the nailer for all models is 2".

- PERFORMANCE CHARACTERISTICS:

  - Extruded bar shall lock membrane, prevent wind pullback.

  - Injection molded EPDM splices to allow thermal expansion of extruded aluminum anchor bar.

  - Fascia shall freely thermal cycle on extruded bar, preventing periodic maintenance.

  - Fascia may be factory modified for true radius application.

- Fascia metal gauge (choose): .040" thick formed aluminum or 24 ga. galvanized steel with Kynar 500 finish.

- Fascia: standard 12'-0" (3.65 m) lengths.

- Extruded bar: Shall be continuous 6063-T6 alloy aluminum at 12'-0" (3.65 m) standard lengths. All bar miters are welded.

- Fasteners: #9 x 2" stainless steel fasteners provided with drivers. No exposed fasteners permitted.

- Exterior fascia finishes: (For aluminum choose) natural mill finish, Kynar 500 from manufacturer’s standard colors, custom color Kynar 500, or clear or color anodized. (For 24 gauge galvanized steel choose) Kynar 500 standard color from manufacturer’s standard colors.

2.03	ACCESSORIES:

- Miters, downspout scuppers, spillout scuppers shall be fabricated by manufacturer.

- Welded base assembly shall be used to maintain watertight integrity.

- Provide matching brick wall cap, downspout, extenders, or other special fabrications as detailed.

III.	PART THREE EXECUTION

3.01	INSPECTION:

- Verify that the roof edging installation will not disrupt other trades. Verify that the substrate is dry, clean and free of foreign matter. Report and correct defects prior to any installation.

3.02	INSTALLATION OF PRESTO-TITE FASCIA SYSTEM:

- Submit product design drawings for review and approval to Architect or Specifier before fabrication.

- Installing contractor shall check as-built conditions and verify the Johns Manville’s roof edge details for accuracy to fit the wall assembly prior to fabrication. The installer shall comply with Johns Manville’s installation guide when setting edging.

- Installer shall use provided fasteners consistent with Johns Manville’s instructions, suitable for the substrate to which it is being installed.

- Install membrane compatible Johns Manville non-curing sealant, as recommended by Johns Manville, under the anchor bar.

END OF SECTION 07710